# Cookie Factory Virtual Model Documentation: Ignition HMI

Contents

Introduction	2

Ignition Gateway	2

Description	2

Connection to OpenPLC	2

HMI in Ignition Designer	8

Description	8

Opening the HMI	8

Ignition Tags	16

Description	16

HMI Tags	16

dv	17

flour_alarm	17

flour_weight	17

hopper_alarm	17

hopper_weight	17

rv_1	17

rv_2	17

rv_3	17

sugar_alarm	18

sugar_weight	18

vacuum	18

vacuum_alarm	18

vacuum_freq	18

vacuum_manual	18

vacuum_rpm	18

# Introduction

The purpose of this document is to aid you in the modification, emulation, or use of the Ignition human machine interface (HMI) for the Cookie Factory Virtual Model, part of CCI subtask 5.1.

While steps to open the HMI project file and to start the Ignition Gateway will be discussed in this document, it is highly recommended that you first complete at least the first 5 steps of the startup guide, created by inductive automation, before proceeding:

https://www.docs.inductiveautomation.com/docs/8.1/getting-started/quick-start-guide

The following document contains a general overview of what a human machine interface (HMI) is, how to configure the ignition gateway to connect to OpenPLC, how to load the project file in the Ignition designer, tags used in the HMI, what a tag is, and descriptions of the different visual components of the HMI as well as how they are influenced by each tag.

Note that this document is in no way complete, because the model itself is not complete. So, check back in the box folder for any updates to the HMI and its documentation, though I will probably let you know when an update occurs.

# Ignition Gateway

## Description

The ignition gateway is a web server that allows you to configure various components of your ignition projects, user permissions, device connections, tag diagnostics, etc. It essentially acts as a control center for all your ignition projects. We will be using the gateway to connect to our OpenPLC device.

## Connection to OpenPLC

Begin by opening your Ignition Gateway, by default it should be using port 8088 on your local machine: http://localhost:8088. If you need more guidance with how to open/set up the Ignition Gateway be sure to check out step 1 and 2 of the start-up guide. On start up you should see the following:

Next, we want to add a Modbus device to our service. Navigate to the configuration tab on the left.

Now you should have the configuration page open, scroll down on the left side of the screen until you reach the OPC UA section. Next, click on device connections.

Now you’ll see an empty screen, we’ll want to add an OPC UA device that represents our OpenPLC device. Select “Create new Device…”

You should see multiple device options to choose from, scroll down until you see “Modbus TCP” and select it. Scroll down and select “Next.”

Now you should be able to select your device name, the hostname/port to connect to, and adjust any advanced settings. Match the hostname/port to what is set as your hostname/port in OpenPLC runtime, by default it should be 127.0.0.1 (localhost) and 502 for the port number.

Next, we need to change the indexing setting for Ignitions Modbus addressing. Ignition uses 1-based indexing for Modbus addresses so without changing this setting all our addresses will be 1 register off their actual value. To change this, check the “Show advanced properties” box near the bottom of the page. Then scroll down to “Zero-based Addressing” and check the box to true.

After specifying all the necessary properties, scroll down to the bottom of the page and select “Create New Device.”

Congratulations! You’ve now created a new device to represent your OpenPLC runtime, if your hostname and port are set correctly you should see your new device and that its status is “Connected.” Now that we have created a new device, and it’s connected to our OpenPLC runtime instance, we need to properly address any relevant tags for our Ignition HMI. To do this click on the “Addresses” option under “More.”

Now you should see a blank addresses page that allows you to add rows or import a configuration. Click the “Choose File” button and upload the latest configuration file named modbus-config.csv in the directory of this document on Box. Then click “Import Configuration.”

Now you should see multiple rows of different addresses that are set to different Modbus addresses. The prefix and start/ends are set to be reminiscent of OpenPLC’s addressing system; however you should be able to change the prefix of each address as you please.

Click save at the bottom of the page and now we are ready to open the HMI in Ignition Designer.

### Cookie tag provider

Another component that we need to set up is the cookie tag provider. Begin by navigating to Config->Tags->Realtime.

Next select “Create new Realtime Tag Provider”

Select next

Then name the new provider “cookie” and leave all the other settings to their default settings.

# HMI in Ignition Designer

## Description

A human machine interface (HMI) is a system that allows people to interact and control different physical processes, systems, or machines. Typically, an HMI allows the user to see a simplified view of different systems and their relative information.  Multiple HMIs may exist within a single physical facility, often encompassing different parts of facilities operations. Currently, the project file uploaded to box contains the HMI that represents the ingredient allocation process of the Cookie Factory Model.

## Opening the HMI

After installing/opening the designer launcher, as outlined in step 5 of the startup guide, you will be able to add a designer using the default gateway and your personal device as shown below. If you need more assistance with this step be sure to check out the following page:
https://www.docs.inductiveautomation.com/docs/8.1/launchers-and-workstation##adding-applications

Select your new designer and open it. You will be asked to log in, the default log in is username: admin, and password: password, otherwise it will be the credentials you first created during Ignition installation.

Next you will see a page that has all your active projects, yours will be blank. To import the HMI for the cookie factory, select import project as shown below.

This will open your file explorer where you can select your project zip file. For the cookie factory download and select the project zip file located in box named ManufacturingModel_CookieFactory_2025-12-30_1155 in the same directory as this document.

Next you should see a screen showing all the project configurations, leave these as is and then select “Import Project” as shown below, with the default tag provider set to “cookie”.

Now you should see this new project in your list of projects, select “Open” next to the project name.

Congratulations! You have now opened the project file, next we need to import our tags into our tag browser in the bottom left. Select the three dots in the tag browser pane then import tags and then finally direct.

This should open your file browser where you can select the cookie-factory-tags_2025_12_31_1118 json file.

After selecting open you should now see that the tag browser has filled with all the tags included in the Human Machine Interface.

## All About the Designer

After opening the designer and you should see the following:

If you see any errors, such as “Trial Expired” or the tag values in red font, be sure to reset your trial in the ignition gateway or ensure you are connected to the OpenPLC runtime instance.

There are four main windows in Ignition Designer that you should direct your attention to. First there is the “Project Browser” which shows all project components from scripts to visual elements, alarms, reports, and database queries.

Next is the “Tag Browser” where you can see all the tags included in the HMI, these numbers will change as the python simulation runs in accordance with an operator’s inputs into the HMI.

Next, you have the main window which will show you the HMI in it’s current state. The main window can be used to show you three separate views; “FinishedGood,” “Resource Allocation,” and “ProductionArea.” Most components in each view are tied to a specific tag, we’ll explain what each tag does and what component they are tied to. The tag values will change in accordance with the HMI and these changes will also be reflected in OpenPLC’s monitor.

To manipulate these values, we first must enable read/write mode and then preview mode. This will allow us to click different components and change the values accordingly.

Congratulations! Now you should understand how to load a project into Ignition Designer and manipulate said project.

# Ignition Tags

## Description

Tags are points of data that are statically or dynamically changed. Tags can be pulled from expressions, databases, or through Open Platform Communications (OPC) addresses. In our case tags are pulled from our OpenPLC Modbus device that is connected to Ignition’s default OPC Unified Architecture (UA) server.

## HMI Tags

Although our model has dozens of tags defined in its OpenPLC program, our HMI only needs access to tags that directly influence the operators’ control over the model’s systems. For this reason, our HMI has ~65 tags, each of these tags corresponds to at least one visual element in our HMI. Now we will go over each tag, what they represent, and what visual element they correspond to. Some tags are not implemented, these tags are highlighted.

### dv

This tag represents the status of the diverter valve, if the value is set to 0, the valve is set to siphon sugar, if the value is set to 1, the valve is set to siphon flour.

### flour_alarm

This tag represents the status of the flour alarm, if the value is set to 0, then the flour_weight tag has a value greater than 1000, if the value is set to 1, then the flour_weight tag has a value less than 1000.

### flour_weight

This tag represents the total weight of the material in the flour silo.

### hopper_alarm

This tag represents the status of the hopper alarm, if the value is set to 0, then the hopper_weight tag has a value greater than 100, if the value is set to 1, then the hopper_weight tag has a value less than 100.

### hopper_weight

This tag represents the total weight of the material in the hopper

### rv_1

This tag represents the rotary valve that controls the input of flour into the pneumatic conveying system. The rotary valve is open and spinning if the value of rv_1 is 1. The rotary valve is closed and not spinning if the value of rv_1 is 0.

### rv_2

This tag represents the rotary valve that controls the input of sugar into the pneumatic conveying system. The rotary valve is open and spinning if the value of rv_2 is 1. The rotary valve is closed and not spinning if the value of rv_2 is 0.

### rv_3

This tag represents the rotary valve that controls the output of material from the hopper into the mixer.  The rotary valve is open and spinning if the value of rv_3 is 1. The rotary valve is closed and not spinning if the value of rv_3 is 0.

### sugar_alarm

This tag represents the status of the sugar alarm, if the value is set to 0, then the sugar_weight tag has a value greater than 1000, if the value is set to 1, then the sugar_weight tag has a value less than 1000.

### sugar_weight

This tag represents the total weight of the material in the sugar silo.

### vacuum

This tag represents the on/off state of the vacuum for the pneumatic conveying system. If the value is 1, then the pneumatic conveying system will move material. If the value is 0, then the pneumatic conveying system will not move material.

### vacuum_alarm

This tag represents the status of the vacuum alarm. If the value is set to 0, then the vacuum is performing at a reasonable speed for the material being conveyed. If the value is set to 1, then the vacuum is underperforming in speed for the material being conveyed leading to saltation.

### vacuum_manual

This tag represents the state of the frequency control for the vacuum. If the value is set to 0, then the vacuum is set to an automatic frequency based on the material being conveyed. If the value is set to 1, then the vacuum’s frequency can be manipulated by the operator.

### vacuum_rpm

This tag represents the rpm of the vacuum’s motor. The higher the rpm, the faster material is conveyed.

### trough_transfer

This tag represents the transfer of raw material from the mixer to the wirecutter trough

### add_ingredients

This tag when triggered adds the non-bulk ingredients to the mixer, whether it be chocolate chips, brown sugar, or some other ingredient besides flour and sugar.

### mixer

This tag represents whether the mixer is on or off, basically when it’s spinning or not spinning

### mixer_rpm

This tag represents how fast the mixer is spinning and mixing the dough

### mixer_weight

This tag represents how much dough is in the mixer; there can be a maximum of 60 pounds of dough in the mixer at one time.

The location of all visual elements and their relevant tags can be seen below for the “ResourceAllocation” view.

### wirecutter

This tag represents the on and off state of the wirecutter, which cuts the cookie dough into puck size. The wirecutter is cutting the dough if the value is set to 1, if the value Is set to 0 it is not cutting.

### trough_weight

This tag represents the total weight of dough in the trough currently and decreases as the wirecutter cuts.

### wirecut_cpm

This tag represents the wirecutters cuts per minute.

### papercut_cpm

This tag represents the paper cutter cuts per minute.

### paper_cutter

This tag represents the on and off state of the paper cutter, which cuts the pucks of dough on parchment paper into different sheets. The paper cutter is cutting the paper if the value is set to 1, if the value Is set to 0 it is not cutting.

### conveyor

This tag represents the on and off state of the conveyor belt which carries the sheets of dough pucks through the nitrogen tunnel to the finished good area. If the value is 1 the conveyor is moving, if the value is 0 then the conveyor is no longer moving.

### conveyor_fpm

This tag represents the conveyor feet per minute, or how fast the cookies are traveling down the line. This tag is always set to 0 when the conveyor is off.

### tunnel_temp

This tag represents the temperature of the nitrogen tunnel, it fluctuates based on the input of nitrogen and how many fans are on, particularly if the exhaust fan is on.

### gv_1

This tag represents the open and closed state of the gas valve which regulates the nitrogen in the tunnel freezing system. The gas valve is open if the value is set to 1 and close if it is set to 0.

### fan_1

This tag represents the on and off state for fan number 1. The blades of the fan are spinning if the value is set to 1 or not spinning if the value is set to 0.

### fan_2

This tag represents the on and off state for fan number 2. The blades of the fan are spinning if the value is set to 1 or not spinning if the value is set to 0.

### fan_3

This tag represents the on and off state for fan number 3. The blades of the fan are spinning if the value is set to 1 or not spinning if the value is set to 0.

### exhaust_fan

This tag represents the on and off state for the exhaust fan. The blades of the fan are spinning if the value is set to 1 or not spinning if the value is set to 0.

### nitrogen_volume

This tag represents the total amount of nitrogen currently in the nitrogen tank.

The location of all visual elements and their relevant tags can be seen below for the “ProductionArea” view.

### conveyor_1

This tag represents the on and off state for conveyor_1, the conveyor from the box maker to the bag inserter. If the value is set to 1 then the conveyor is on if the value is set to 0 then the conveyor is off.

### conveyor_2

This tag represents the on and off state for conveyor_2, the conveyor from the bag inserter to the packing area. If the value is set to 1 then the conveyor is on if the value is set to 0 then the conveyor is off.

### conveyor_3

This tag represents the on and off state for conveyor_3, the conveyor from the packing area to the box taper. If the value is set to 1 then the conveyor is on if the value is set to 0 then the conveyor is off.

### conveyor_4

This tag represents the on and off state for conveyor_4, the conveyor from the box taper to the palletizer. If the value is set to 1 then the conveyor is on if the value is set to 0 then the conveyor is off.

### conveyor_5

This tag represents the on and off state for conveyor_5, the conveyor from the palletizer to the forklift pick up spot. If the value is set to 1 then the conveyor is on if the value is set to 0 then the conveyor is off.

### conveying_1

This tag represents the conveying state for conveyor_1. If the value is set to 1 then the conveyor moving if the value is set to 0 then the conveyor is not moving. If conveyor_1 is set to 0 then conveying_1 is set to 0 as well.

### conveying_2

This tag represents the conveying state for conveyor_2. If the value is set to 1 then the conveyor moving if the value is set to 0 then the conveyor is not moving. If conveyor_2 is set to 0 then conveying_2 is set to 0 as well.

### conveying_3

This tag represents the conveying state for conveyor_3. If the value is set to 1 then the conveyor moving if the value is set to 0 then the conveyor is not moving. If conveyor_3 is set to 0 then conveying_3 is set to 0 as well.

### conveying_4

This tag represents the conveying state for conveyor_4. If the value is set to 1 then the conveyor moving if the value is set to 0 then the conveyor is not moving. If conveyor_4 is set to 0 then conveying_4 is set to 0 as well.

### conveying_5

This tag represents the conveying state for conveyor_5. If the value is set to 1 then the conveyor moving if the value is set to 0 then the conveyor is not moving. If conveyor_5 is set to 0 then conveying_5 is set to 0 as well.

### boxing

This tag represents whether the box maker is making a box or not. If the value is set to 1 then the box maker is making a box, if the value is set to 0 then the box maker is not making a box.

### box_maker

This tag represents the on and off state of the box maker. If the value is set to 1 then the box maker is on, if the value is set to 0 then the box maker is off.

### bagger

This tag represents the on and off state of the bag inserter. If the value is set to 1 then the bag inserter is on, if the value is set to 0 then the bag inserter is off.

### worker_present

This tag shows whether the worker is present at the packing area or not. If the value is set to 1 then the worker is present and is packing if there are cookie sheets/boxes available, if the value is set to 0 then the worker is not present at the packing station.

### taper

This tag represents the on and off state of the box taper. If the value is set to 1 then the box taper is on, if the value is set to 0 then the box taper is off.

### palletizer

This tag represents the on and off state of the palletizer. If the value is set to 1 then the palletizer is on, if the value is set to 0 then the palletizer is off.

### palletizer_grabbing

This tag represents whether the palletizer is grabbing a box or not. If the value is set to 1 then the palletizer is grabbing a box, if the value is set to 0 then the palletizer is not grabbing a box.

### palletizer_moving

This tag represents whether the palletizer is moving or not. If the value is set to 1 then the palletizer is moving, if the value is set to 0 then the palletizer is not moving.

### palletizer_rotation

This tag represents the angle at which the palletizer arm is rotated. The angle changes based on if the palletizer is moving and/or grabbing a box.

### roe_1

This tag represents a sensor on the palletizer which is set to true when the palletizer_rotation is equal to 130 degrees.

### roe_2

This tag represents a sensor on the palletizer which is set to true when the palletizer_rotation is equal to 180 degrees.

### roe_3

This tag represents a sensor on the palletizer which is set to true when the palletizer_rotation is equal to 225 degrees.

The location of all visual elements and their relevant tags can be seen below for the “FinishedGood” view.